| Calibration technique | Explanation | Equation for Systolic BP | Equation for Diastolic BP |
| --- | --- | --- | --- |
| “Ignore the Change” | - Take RZS value at visit 1 - Take AOD value at visit 2 when available; take RZS value at visit 2 otherwise - Take AOD value at visit 3 | -- | -- |
| Ordinary Least Squares regression | - Determine OLS regression calibration equation, using visit 2 BPCS cohort - Calibrate RZS to AOD at visit 1 - Take AOD value at visit 2 when available; calibrate RZS to AOD at visit 2 otherwise - Take AOD value at visit 3 | SBP = 0.93*RZS + 9.81 | DBP = 0.83*RZS + 10.40 |
| Average difference | - Determine AOD-RZS difference, using visit 2 BPCS cohort - Offset RZS to AOD at visit 1 - Take AOD value at visit 2 when available; offset RZS to AOD at visit 2 otherwise - Take AOD value at visit 3 | SBP = RZS + 1.09 | DBP = RZS + -2.44 |
| Deming regression | - Determine Deming regression calibration equation, using visit 2 BPCS cohort - Calibrate RZS to AOD at visit 1 - Take AOD value at visit 2 when available; calibrate RZS to AOD at visit 2 otherwise - Take AOD value at visit 3 | SBP = 1.04*RZS + -4.31 | DBP = 1.04*RZS + -6.07 |
| Robust regression | - Determine robust regression calibration equation, using visit 2 BPCS cohort - Calibrate RZS to AOD at visit 1 - Take AOD value at visit 2 when available; calibrate RZS to AOD at visit 2 otherwise - Take AOD value at visit 3 | SBP = 11.04 + RZS + -0.08*RZS | DBP = 10.36 + RZS + -0.17*RZS |